This is for COMAND 2.5 (Fitted to S/CL up until end of MY2002) European and US versions - NOT Japan 
Note that connector A is not wired in the standard DIN manner. Connector B (the loudspeaker connector is) 

	Comand 2.5 A connector

	1
	+12 V 
	5
	+12v switched out to aerial 

	2
	Ground 
	6
	spare 

	3
	Phone Mute 
	7
	spare 

	4
	+12 v 
	8
	Ground 


	Comand C1 connector

	1
	TV Audio Shield 
	5
	TV Audio Gnd 

	2
	TV Audio Left 
	6
	TV Audio right 

	3
	spare 
	7
	spare 

	4
	spare 
	8
	spare 


	Comand C2 connector

	1
	K diagnosis line 
	10
	spare 

	2
	To Plug E/21 
	11
	CAN-B Shield 

	3
	To Plug E/22 
	12
	To Plug E/9 

	4
	To Plug E/23 
	13
	To Plug E/10 

	5
	To Plug E/26 
	14
	To Plug E/13 

	6
	To Plug E/25 Shield 
	15
	CAN-B L 

	7
	D2B Wakeup 
	16
	CAN-B H 

	8
	spare 
	17
	RB-CAN L (TV & E/3) 

	9
	spare 
	18
	RB-CAN H (TV & E/2) 


	Comand D connector (5 pin round)

	1
	TV Video Red 
	4
	TV Video Sync 

	2
	TV Video Green
	5
	TV Video Gnd/Shield 

	3
	TV Video Blue 
	
	


	Comand E connector (Nav Unit)

	1
	Gnd 
	14
	+12V 

	2
	To C2/18 RB-CAN H 
	15
	spare 

	3
	To C2/17 RB-CAN L 
	16
	spare 

	4
	spare 
	17
	spare 

	5
	spare 
	18
	spare 

	6
	spare 
	19
	To CTEL Wakeup 

	7
	Speed Signal Left 
	20
	Speed Signal Right 

	8
	spare 
	21
	To C2/2 (shield for 9,10,22,23) 

	9
	To C2/12 
	22
	To C2/3 

	10
	To C2/13 
	23
	To C2/4 

	11
	spare 
	24
	spare 

	12
	spare 
	25
	To C2/6 (shield for 13/26) 

	13
	To C2/13 
	26
	To C2/5 


